EEG Temporal Detection Cheat Sheet

Objective: Identify the exact start (onset) and stop (offset) times of physiological artifacts using purely temporal EEG signal characteristics.

1. The Grading Matrix

Your inspections will be evaluated based on these strict tolerance windows. Always classify the artifact duration first to know your margin of error.

| Category | Artifact Labels | Typical Duration | Onset Tolerance | Offset Tolerance |
| --- | --- | --- | --- | --- |
| Micro-Events | ver_eyem (Vertical Saccade) | < 1 second (~0.25 - 0.4s) | ± 200 ms | ± 300 ms |
| Medium Blocks | tongue, swallow | 1 to 3 seconds (Tongue bursts: ~0.6s) | ± 500 ms | ± 1000 ms |
| Long Blocks | hor_headm, ver_headm, eyebrow, chew | 4 to 15 seconds | ± 1.5 seconds | ± 2.0 seconds |
| Macro States | blink (Task), close_base, open_base | > 30 seconds | ± 3.0 seconds | ± 3.0 seconds |

2. Temporal Signatures (What to look for)

Since spatial channels are hidden, use these temporal shape profiles to classify the artifact type:

The "Spike" (Micro-Events)

- Profile: Sudden, massive increase in amplitude followed by an immediate return to baseline.

- Look for: ver_eyem. Sharp, distinct, and isolated.

The "High-Frequency Burst" (Medium Blocks)

- Profile: A dense, jagged cluster of high-frequency noise. Unlike a single eye movement, muscle recruitment (EMG artifact) builds up and trails off.

- Look for: swallow (lasts ~2s with a distinct peak and taper), tongue (rapid repeating bursts).

The "Sustained Storm" (Long Blocks)

- Profile: A prolonged period where the baseline voltage shifts dramatically, or a thick, sustained band of muscle noise obscures the normal EEG rhythm.

- Look for: eyebrow (sustained tension), chew (rhythmic high-amplitude crushing), headm (slow, rolling waves combined with muscle noise).

The "Epoch Boundary" (Macro States)

- Profile: Massive chunks of the timeline.

- Look for: blink (continuous, metronomic high-amplitude spikes lasting ~60 seconds). close_base (stable, low-amplitude resting state).

3. Recommended Detection Workflow

When scanning the data, follow this sequence:

- Isolate Macro States First: Find the massive 60-second task blocks (like the continuous blinking task). Remove or flag these massive chunks so they don't trigger false positives for your micro-event detectors.

- Define the Baseline: Calculate the moving average and standard deviation of the "quiet" periods (close_base or open_base).

- Set Amplitude Thresholds: Use your baseline to set standard deviation multipliers (e.g., flag any timestamp where the voltage exceeds 3 standard deviations from the resting mean).

- Measure the Duration: Once an anomaly triggers your threshold, calculate Stop Time - Start Time. Use this duration to classify the event into Category 1, 2, 3, or 4.